DGSA Examination

March 2026

CILT Markers’ report

Overview

The pass rate for both papers was particularly high in this examination. There was a high number of refresher and of repeat initial candidates which may have contributed to the high pass rate.

Paper 1

Question 1 on classification was well answered, candidates worked their way through this question without any difficulty.

Question 2 was on limited quantities – a few candidates omitted the orientation arrows or to put them on two opposite sides.

Question 3 was a short question on fixed penalties and caused some confusion for a few candidates.

Question 4 was also a short question on driver duties and was very well answered.

Question 5 on driver training did not create many issues.

Question 6 had a number of deliberate errors in a transport document which most candidates recognized.

Question 7 had two marks which can be seen on road tankers. Almost all candidates could not recognize the symbol for the earth fitting, which was a concern as earthing and the dissipation of static electricity is a critical safety element for tankers particularly when handling flammable liquids and gases.

Question 8 on duties and responsibilities was well answered.

Question 9 on IBCs was the worst answered question in paper 1. Many candidates did not demonstrate an understanding of the details regarding the transport of out of test IBCs.

Question 10 was a short question on definitions which was well answered.

Paper 2

The compulsory case study involved the transport of an elevated temperature substance in tanks and was well answered.

Candidates were asked to write a new transport document for the empty return journey, but a number answered by altering the original transport document.

Most candidates opted for Case Study 2 which involved a consignment of IBCs on an International Journey. Again this case study was well answered with a high overall mark similar to previous marks for this case study.

Two aspects of the case study which were not well covered related to transport in an open flatbed vehicle and what to do after a spill of the substance. These are very practical questions which a DGSA is likely to encounter in their work and greater emphasis should be placed on these aspects in training.

A small number of candidates attempted Case Study 3 on the transport of a polymerizing liquid. Polymerising substances were introduced into ADR and other modal regulations a number of years ago following a very serious incident on a ship named MSC Flaminia which caught fire due to mishandling of these products in Mid Atlantic in 2012 with a number of fatalities.

One of the candidates (initial) almost achieved full marks in this case study.

As stated in previous markers’ reports, we note that candidates tend to avoid case studies involving such topics as lithium batteries, radioactives, class 5.1 and so on. Let us again advise training providers and students that as successful candidates are awarded a DGSA certificate covering all classes then they should expect to be examined in all classes and that the compulsory case study will involve one of the so called minority classes at a future exam.

The methodology for answering a case study is the same irrespective of the class. The case study is a test of the students ability to identify the subject matter in ADR and then navigate the book to find the answers.